# Project Documentation

## Number Plate Detection – YOLOv8

## Purpose of This Document

This document explains the technical workflow, dataset usage, training logic, and deployment steps of the project.

## System Overview

The system detects number plates in images using a YOLOv8-based object detection model. The workflow includes image collection, annotation, model training, evaluation, and deployment preparation.

## Dataset Usage

Toy car images were collected manually using three toy car models with generated number plates. Real car images were sourced from a Kaggle dataset to improve generalization.
Training images: 431
Validation images: 39

## Annotation Pipeline

Tool: CVAT
Task: Object Detection
Annotations: Bounding boxes
Export format: YOLO (.txt files with normalized coordinates)

## Model Selection

YOLOv8n was selected due to its lightweight architecture, fast inference speed, and suitability for embedded deployment.

## Training Workflow

Training was performed in Google Colab.
Input size: 640x640
Epochs: 80
Pretrained weights: yolov8n.pt

## Evaluation Strategy

Metrics used include Precision, Recall, mAP@50, and mAP@50–95.

## Iterations and Design Decisions

Initial training using only toy car images caused overfitting. Real car images were added to improve robustness and generalization.

## Inference Pipeline

The best trained model (best.pt) is loaded and inference is performed on test images with confidence thresholding and bounding box visualization.

## Deployment Preparation

The trained model is exported to IMX format and converted into RPK format for deployment on Raspberry Pi with Sony IMX500.

## Known Limitations

Limited dataset size, only three toy car models, and detection-only scope (no OCR).

## Possible Improvements

Increase dataset diversity, improve robustness to lighting and motion, and integrate OCR for full number plate recognition.

## Summary

This document describes the technical workflow and design decisions of the number plate detection system.